Create an Ubuntu VM on AWS and install Apache, ending with testing the default Apache landing page.
This works on AWS Free Tier / lowest cost.

4

1️⃣ Sign in to AWS

- Go to AWS Management Console

- Sign in with your AWS account

- At the top-right, select a Region
👉 Cheapest & safest: US East (N. Virginia)

2️⃣ Launch an Ubuntu EC2 instance

AWS Console → Services → EC2 → Instances → Launch instance

Step A — Name & OS

- Name: ubuntu-apache-lab

- AMI: Ubuntu Server 22.04 LTS (64-bit x86)
(Provided by Canonical, Free Tier eligible)

Step B — Instance type

- t2.micro (Free Tier)
or t3.micro (very cheap, faster)

Step C — Key pair (SSH access)

- Click Create new key pair

- Name: aws-ubuntu-key

- Type: RSA

- Format: .pem

- Download and keep it safe

Step D — Network settings (IMPORTANT)

Click Edit

Security Group → Inbound rules:

| Type | Protocol | Port | Source |
| --- | --- | --- | --- |
| SSH | TCP | 22 | My IP |
| HTTP | TCP | 80 | Anywhere (0.0.0.0/0) |

⚠️ HTTP (80) must be allowed or Apache will not load.

Step E — Storage

- 8 GB (default) – keep as is

Click Launch instance

3️⃣ Connect to the Ubuntu VM (SSH)

Wait until instance status = Running

EC2 → Instances → select your instance → Connect

Option A — Browser (easiest)

- Choose EC2 Instance Connect

- Click Connect

Option B — From your own terminal (PowerShell / WSL / macOS)

chmod 400 aws-ubuntu-key.pem

ssh -i aws-ubuntu-key.pem ubuntu@<PUBLIC-IP>

4️⃣ Update Ubuntu

Inside the VM:

sudo apt update

sudo apt upgrade -y

5️⃣ Install Apache Web Server

sudo apt install apache2 -y

Start & enable Apache:

sudo systemctl start apache2

sudo systemctl enable apache2

Check status:

systemctl status apache2

You should see active (running).

6️⃣ Test Apache default landing page 🎉

Step A — Get Public IP

EC2 Console → Instance → copy Public IPv4 address

Step B — Open browser

http://<PUBLIC-IP>

✅ You should see:

“Apache2 Ubuntu Default Page”

That confirms:

- EC2 VM is running

- Apache is installed

- Security Group allows HTTP

7️⃣ (Optional) Verify Apache files

Default web root:

ls /var/www/html

Default page file:

cat /var/www/html/index.html

8️⃣ (Optional) Replace default page

sudo nano /var/www/html/index.html

Example:

<h1>Hello from AWS Ubuntu Apache</h1>

<p>Student lab successful.</p>

Save → refresh browser.

9️⃣ Cost control (VERY IMPORTANT for students)

When finished:
EC2 → Instance → Stop

✅ Status must show Stopped (not Running)
💸 Stopping saves most charges

🧠 What you have learned (lab-ready)

- Launching an Ubuntu VM on AWS

- Configuring security groups

- Installing & managing Apache

- Verifying web services via browser